# SECTION 081316

#### PART 1 – GENERAL

1.01 GENERAL SCOPE

- Furnish all labor, materials, tools and equipment required to complete the hinged glass door project as shown on the architectural drawings and as specified in 081316 and other related sections.

- Renovation Projects: Removal of existing doors or vents as required by the project documents.

- New Construction Projects: Position the new doors and attachment systems into the openings as indicated by the architectural drawings.

- Provide factory glazed, fully thermally broken aluminum hinged doors and all additional components and systems as required by this specification and the architectural drawings.

1.02 INDUSTRY REFERENCES

AAMA - American Architectural Manufacturers Association

AAMA 502-02 "Voluntary Specification for Field Testing of Windows and Sliding Glass Doors"

AAMA 2603-05 "Voluntary Specification, Performance Requirements and Test Procedures for Pigmented Organic Coatings on Aluminum Extrusions and Panels"

AAMA 2604-05 "Voluntary Specification, Performance Requirements and Test Procedures for High Performance Organic Coatings on Aluminum Extrusions and Panels"

AAMA 2605-05 "Voluntary Specification, Performance Requirements and Test Procedures for Superior Performing Organic Coatings on Aluminum Extrusions and Panels"

AAMA 701&702-04 "Voluntary Specification for Pile Weatherstripping & Replaceable Fenestration Weatherseals"

AAMA 800-05 "Voluntary Specifications and Test Methods for Sealants"

AAMA 902-99 "Voluntary Specification for Sash Balances"

AAMA 1503-98 "Voluntary Test Method for Condensation Resistance of Windows, Doors, and Glazed Wall Sections"

AAMA CW-10-04 "Care and Handling of Architectural Aluminum from Shop to Site"

ANSI - American National Standards Institute

ANSI/AAMA/WDMA/CSA 101/I.S.2-97 "Standard/Specification for Windows, Doors, and Unit Skylights”

ASTM - American Society for Testing and Materials

ASTM C 1036-06 "Standard Specification for Flat Glass"

ASTM C 1048-04 "Standard Specification for Heat-Treated Flat Glass"

ASTM E 90-04 "Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements"

ASTM E 283-04 "Standard Test Method for Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors"

ASTM E 330-02 "Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference"

ASTM E 331-00 "Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors and Curtain Walls by Uniform Static Air Pressure Difference"

ASTM E 547-00 "Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors and Curtain Walls by Cyclic Static Air Pressure Differential"

ASTM E 774-97 "Standard Specification for the Classification of the Durability of Sealed Insulating Glass Units"

1.03 INDICATE PRODUCTS FURNISHED BUT NOT INSTALLED IN THIS SECTION

- (#) extra sets of glazed sash for each window size will be supplied and delivered to the project at job completion for storage and future needs.

- (#) extra screens for each window size will be supplied and delivered to the project at job completion for storage and future needs.

1.04 INDICATE PRODUCTS INSTALLED BUT NOT FURNISHED IN THIS SECTION

- Install (#) supplied by (Name) into the new windows in the (Location) rooms as directed.

- Reinstallation of air conditioners in (#) windows as directed by the architect.

- Reinstall existing air-vents or ducts through (#) windows.

- Reinstall existing window treatments and/or blinds at each window.

1.05 RELATED SECTIONS

- Section 07190 - Vapor and Air Barriers

- Section 07900 - Sealants

1.06 QUALITY ASSURANCE

A.  	 Provide test reports from an AAMA accredited laboratory certifying the performance as specified in section 1.07 of this specification.  Valid test reports should be no more than four years old.

B.   	To ensure that the windows provided for this project are exactly the same as the sample sent to the testing lab for certification, the manufacturer should be an active member in the AAMA Certification Program. All bidders must provide a current NOTICE OF PRODUCT CERTIFICATION from the AAMA administrator / validator, Associated Laboratories Inc. (ALI).

C.   	Pre-Bid Qualifications: The basis of this specification is Champion Window and Door products located at 140 Eileen Way, Syosset, NY 11791.  All other bids must pre-qualify for their products.

1.07 PRE-QUALIFICATION

Submit for pre-bid qualification approval ten days prior to the bid opening:

- A sample window matching the project criteria with the only exception being the color.

- Current test reports that conform to AAMA/WDMA/CSA 101/I.S.2/A440-08 should be less than 4 years old.

- Full size product and accessory details.

- Complete product specifications.

- The current NOTICE OF PRODUCT CERTIFICATION from the AAMA administrator validator, Associated Laboratories Inc. (ALI).

- Pre-qualified "equal" products will be confirmed in a written addendum.

1.08 SYSTEM DESCRIPTION

A.   	AAMA Designation: ATW-AW-PG75

B.    	Door: 4 1/2" frame depth; extruded aluminum with integral structural polyurethane thermal break in the frame and sash members; equal-leg frame; finish factory-applied; frames and sash factory-assembled.

C.    	Configuration: Hinged Glass Door [Optional: door panel, transom and sidelite frames factory-assembled and factory-glazed configurations can be either field or factory mulled together.]  All doors are pre-hung single [Optional French double door panel in door frame; out-swing]; [Optional right hand indicates hinges on left jamb when outside looking in] [Optional left hand indicates hinges on right jamb when outside looking in].

D.    	Glazing: 1" insulating glass units; glass and panel descriptions in paragraph 2.04; factory-glazed..

1.09 PERFORMANCE REQUIREMENTS

- Conformance to ATW-AW-PG75 specifications in AAMA/WDMA/CSA 101/I.S.2/A440-08 when tests are performed on the prescribed 48" x 96" minimum test size with the following test results:

  - Air Infiltration: The window should be subjected to an air infiltration test in accordance with ASTM E 283. Air infiltration shall not exceed 0.09 cfm/ft2 when tested at a static air pressure difference of 6.24 psf.

  - Water Penetration: The window shall be subjected to a water resistance test in accordance with ASTM E 331 and ASTM E 547. There shall be no leakage at a static pressure of 12.12 psf.

  - Uniform Structural: maximum of .4% deformation per member in accordance with ASTM E 330 at 112.85 psf.

  - Forced entry resistance as per ASTM F 588 = Pass.

# PART 2 – PRODUCTS

2.01 APPROVED MANUFACTURER

Champion 400 ATW-AW-PG75

# 2.02 MATERIALS

A.    	Aluminum: Aluminum should be of commercial quality and of proper alloy and temper for window construction, free from defects impairing strength and durability.  All extruded sections should be of 6063-T5 or 6063-T6 and should have a minimum ultimate tensile strength of 22,000 psi and a yield of 16,000 psi.

B.   	Door Members: Fixed frames should have a nominal wall thickness of not less than 0.125”.

C.   	Fasteners: All screws and other miscellaneous fastening devices incorporated in the product should be of aluminum, stainless steel or other corrosion-resistant material(s) compatible with aluminum and of sufficient strength to perform the functions for which they are used.  Cadmium zinc-plated steel, where used, should be in accordance with ASTM A 165 or B 633.  Nickel or chrome-plated steel, where used, should be in accordance with ASTM B 456.

D.   	Hardware: Hardware having component parts which are exposed should be of aluminum, stainless steel or other non-corrosive material(s) compatible with aluminum and of sufficient strength to perform the functions for which they are used.  Cadmium or zinc-plated steel, where used, should be in accordance with ASTM A 165 or B 633.  Nickel chrome-plated steel, where used, should be in accordance with ASTM B 456.  Hinged doors should have a three point locking mechanism and thumb turn interior locking mechanism.  Handle and base plate should be brushed a silver finish. [Optional finishes available upon request.]

E.    	Weather-stripping: Weather-stripping should meet the requirements of the specifications as detailed in the appropriate test report.  All weather-stripping should be installed in specially extruded ports and secured to prevent movement and shrinkage.  Adhered weather-stripping should not be allowed.

2.03 FABRICATION

A.    	Assembly: The hinged glass doors should be assembled in a secure and professional manner to perform as herein specified and to assure neat and weather tight construction.  All corners at the head are to be mitered, keyed, staked and sealed with seam sealer.  All main framing joints should be sealed with sealants meeting the requirement of AAMA 803 or 809.

B.   	Thermal Barrier: All main frames and ventilators should be thermally broken by the poured and debridged method.  The thermal barrier material should be of polyurethane fill with ultimate tensile strength to meet or exceed ASTM D 638.

C.    	Mullions: Mullions should be provided as indicated on the drawings and should be of the same size and type to ensure the structural integrity of the proposed hinged glass door system.

D.    	Glazing: The hinged glass doors and fixed areas should be glazed with 1" sealed insulated tempered 3/16” safety glazing.

E.    	Finish: The exposed surfaces of the aluminum members should be clean and free from serious surface blemishes.

F.    	Frame Style: Box frame with closed back thermally broken.

2.04 GLASS AND GLAZING MATERIALS

- Construction: Provide hermetically sealed insulating glass units.  All aluminum spacers will be continuous with bent corners and containing a dual seal of polyisobutylene and silicone.  The 1” I.G. unit should contain desiccant filled into the aluminum spacer.  Plastic corner keys will not be accepted.

B.   	Exterior glass lite

1. Thickness: 3/16" [Optional: varying glass thickness based upon project requirements]

2. Tint: clear Optional: (Grey, Bronze and Green)

3. Type: Tempered Optional: (Laminated)

4. Coating: Optional: (Pyrolitic Low-E on #2 surface)

- Interior glass lite

1. Thickness: 3/16" [Optional: varying glass thickness based upon project requirements]

2. Tint: clear Optional: (Grey, Bronze and Green)

3. Type: Tempered Optional: (Laminated)

4. Coating: Optional: (Pyrolitic Low-E on #3 surface)

D.    Performance

1. Seal durability: Performance to ASTM E 774-00; visible ALI certification for CBA rating level.

2.05 OTHER GLASS AND GLAZING MATERIALS – ALUMINUM INSULATED PANELS

A.   1” insulated panel with 1/8” hardboard backups and smooth aluminum skin on the interior and exterior.  Core will be polystyrene. Options: (Other panel, Spandrel Glass, etc.)

2.06 FINISH ON ALUMINUM EXTRUSIONS

- Metal Application: Apply finish on clean extrusions free from serious surface blemishes or scratches.  Finish exposed surfaces visible when the installed product's operating sash is closed.

B.    	Finish to be PPG Duracron, which meets AAMA 2603. [Optional: Acranar 50% Kynar AAMA 2604, Duranar 70% Kynar AAMA 2605, Duranar XL]

C.    	Quality standard: conforming to AAMA 2603. [Optional: 2604, 2605]

D.    	Pretreatment: five-stage; zinc chromate conversion coating.

E.    	Application: an approved applicator using a factory-based electrostatic spray and oven bake system.

F.    	Coating quantity: one color coat.

G.    	Dry film thickness: minimum .7 mils [Optional: 1.4 mil Acranar, 1.2 mils Duranar] on exposed surfaces, except inside corners and channels.

H.    	Color: chosen from manufacturer's standards. [Optional: custom color to be selected.]

Standard Clear Anodized Finish:

- AAM12C22A31 Class II (A3) Clear Anodized finish (.4mils).

Optional Anodized Finish:

- AAM12C22A31 Class I (A4) Clear Anodized Finish (.7 mils).

- Color: (Light Bronze) (Medium Bronze) (Dark Bronze) (Black)

2.07 AIR CONDITIONERS

- Provide A/C sashes and A/C kits to the following windows.

- [Optional:  Provide A/C sleeves by a designated manufacturer.  Include manufacturer’s name and model number.]

### PART 3 – EXECUTION

3.01 PROJECT SUBMITTALS

Provide project submittals for the following:

- Product Data: Submit manufacturer's specifications, recommendations and standard details for aluminum window units, including certified test laboratory reports as necessary to show compliance with requirements.

- Shop Drawings:  Submit shop drawings, including location floor plans or exterior wall elevations showing all door openings, typical unit elevations and full size detail sections of every typical composite member.  Indicate the type of anchors, hardware, operators and other components not included in manufacturer's standard data.  Include glazing details and standards for factory glazed units.

C.   	Samples:   Submit three samples of each required aluminum finish on a metal panel sample or a 6-inch long section of extrusion.

3.02 DELIVERY, STORAGE AND HANDLING

- Handle all windows and accessories in accordance with AAMA CW-10.

- Protect the windows and accessories from the elements, construction activities and other hazards until the project is complete.

3.03 PROJECT SITE INSPECTION

- Field verify that the existing door openings are within tolerance, plumb, level, clean and provide a solid anchoring surface and substrate.  Also confirm that the openings are in accordance with approved shop drawings.

3.04 INSTALLATION

- Install all of the aluminum doors and accessories with skilled installers in accordance with all of the manufacturer's recommendations, project documents and approved shop drawings.

- All door openings must be covered and secure at the end of each workday.

- Provide the required shims and blocking and fasten the frames to the opening.

- Set each window plumb, level and square, without twisting or bowing the frames.

- Pack fibrous insulation into voids at the door frame perimeter as requested on the architectural drawings.

- Apply sealant at joints and all required metal-to-metal intersections and the window perimeter as required.  Follow the sealant manufacturer’s recommendations listed in their manufacturer’s data sheets.

3.05 DISPOSAL OF DEBRIS

- Remove all garbage off site and legally dispose of existing doors and debris generated from the installation of the new doors.

3.06 OPTIONAL FIELD TESTING

- Conduct all on-site testing of installed units in performance with AAMA publication number 502-90 Voluntary Specification for Field Testing of Windows and Sliding Glass Doors.  Conduct air and water infiltration testing with the window manufacturer, contractor and owner present.

- An AAMA accredited lab will be hired by the owner to perform the required testing.

3.07 ADJUSTMENT AND CLEAN UP

- Adjust all products, sash, vents and hardware after installation as necessary to provide proper operation and a weather tight installation

- Remove any labels and dirt from the door.

# END OF SECTION 081316