# SECTION 085113

#### PART 1 – GENERAL

1.01 GENERAL SCOPE

- Furnish all labor, materials, tools and equipment required to complete the window project as shown on the architectural drawings and as specified in 085113 and other related sections.

- Renovation Projects: Removal of existing windows, sash, or vents as required by the project documents.

- New Construction Projects: Position the new windows and attachment systems into the openings as indicated by the architectural drawings.

- Provide factory glazed, fully thermally broken aluminum windows and all additional components and systems as required by this specification and the architectural drawings.

1.02 INDUSTRY REFERENCES

AAMA - American Architectural Manufacturers Association

AAMA 502-02 "Voluntary Specification for Field Testing of Windows and Sliding Glass Doors"

AAMA 2604-05 "Voluntary Specification, Performance Requirements and Test Procedures for High Performance Organic Coatings on Aluminum Extrusions and Panels"

AAMA 701/702-04 "Voluntary Specification for Pile Weatherstripping & Replaceable Fenestration Weatherseals"

AAMA 800-05 "Voluntary Specifications and Test Methods for Sealants"

AAMA 902-99 "Voluntary Specification for Sash Balances"

AAMA 1503-98 "Voluntary Test Method for Condensation Resistance of Windows, Doors, and Glazed Wall Sections"

AAMA CW-10-04 "Care and Handling of Architectural Aluminum from Shop to Site"

ANSI - American National Standards Institute

ANSI/AAMA/WDMA/CSA 101/I.S.2-97 "Standard/Specification for Windows, Doors, and Unit Skylights”

ASTM - American Society for Testing and Materials

ASTM C 1036-06 "Standard Specification for Flat Glass"

ASTM C 1048-04 "Standard Specification for Heat-Treated Flat Glass"

ASTM E 90-04 "Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements"

ASTM E 283-04 "Standard Test Method for Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors"

ASTM E 330-02 "Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference"

ASTM E 331-00 "Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors and Curtain Walls by Uniform Static Air Pressure Difference"

ASTM E 547-00 "Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors and Curtain Walls by Cyclic Static Air Pressure Differential"

ASTM E 774-97 "Standard Specification for the Classification of the Durability of Sealed Insulating Glass Units"

1.03 INDICATE PRODUCTS FURNISHED BUT NOT INSTALLED IN THIS SECTION

- (#) extra sets of glazed sash for each window size to be supplied and delivered to the project at job completion for storage and future needs.

- (#) extra screens for each window size to be supplied and delivered to the project at job completion for storage and future needs.

1.04 INDICATE PRODUCTS INSTALLED BUT NOT FURNISHED IN THIS SECTION

- Install (#) supplied by (Name) into the new windows in the (Location) rooms as directed.

- Reinstallation of air conditioners in (#) windows as directed by the architect.

- Reinstall existing air-vents or ducts through (#) windows.

- Reinstall existing window treatments and/or blinds at each window.

1.05 RELATED SECTIONS

- Section 07190 - Vapor and Air Barriers

- Section 07900 - Sealants

1.06 QUALITY ASSURANCE

A.   	Provide test reports from an AAMA accredited laboratory certifying the performance as specified in section 1.07 of this specification.  Valid test reports shall be no more than four years old.

B.   	To ensure that the windows provided for this project are exactly the same as the sample sent to the testing lab for certification the manufacturer shall be an active member in the AAMA Certification Program. All bidders must provide a current NOTICE OF PRODUCT CERTIFICATION from the AAMA administrator / validator, Associated Laboratories Inc. (ALI).

C.   	Pre-Bid Qualifications: The basis of this specification is Champion Window and Door products located at 140 Eileen Way, Syosset, NY, 11791.  All other bids must pre-qualify their products.

1.07 PRE-QUALIFICATION

Submit for pre-bid qualification approval ten days prior to the bid opening:

A.	A sample window matching the project criteria with the only exception being the color.

B. 	Current test reports that conform to AAMA/WDMACSA 101/I.S.2/A440-08, less than 4 years old.

C.	Full size product and accessory details.

D. 	Complete product specifications.

E. 	The current NOTICE OF PRODUCT CERTIFICATION from the AAMA administrator validator, Associated Laboratories Inc. (ALI).

F. 	Pre-qualified "equal" products will be confirmed in a written addendum.

1.08 SYSTEM DESCRIPTION

A. 	AAMA Designation: HS-PG-AW65 Series 7500 Horizontally Sliding Window

B. 	Windows: 4 1/8" frame depth; extruded aluminum with integral structural polyurethane thermal break in the frame and sash members;      equal-leg frame; finish factory-applied; frames and sash factory assembled.

C. 	Configuration: XX dual sash (Optional: outside sash with bypass feature) (Optional: XO or OX) horizontal operation.  (Optional: XOX triple)

D. 	The sill tank cover is removable for maintenance with reuseable sill track end caps.

E. 	Aluminum channel limit stops with rubber bumpers (Optional: to limit the travel to any desired opening.) 6” from each jamb.

F. 	Both sash have a raised stainless steel cap track for ease of heavy duty compression bearing roller operation.

- Glazing: 1" insulating glass with structurally glazed silicone exterior perimeter sealant and snap in glazing bead with EPDM wedge gasket.

1.09 PERFORMANCE REQUIREMENTS

1.09 PERFORMANCE REQUIREMENTS

- Conformance to HS-PG-AW65 specifications in AAMA/WDMACSA 101/I.S.2/A440-08 when tests are performed on the prescribed 99" x79" minimum test size with the following test results:

  - Air Infiltration: maximum 0.17 cfm/ft2 of sash perimeter when tested per ASTM E 283 at a static air pressure difference of 6.24 psf.

  - Water Penetration: The window shall be subjected to a water resistance test in accordance with ASTM E 547 and ASTM E 330. There shall be no leakage at a static pressure of 12.12 psf.

  - Uniform Deflection: No more than L/175 when tested per ASTM E 330-90 at a static air pressure difference of 65.20 psf.

  - Uniform Structural: maximum of .2% deformation per member in accordance with ASTM E 330 at 97.81 psf.

  - Life Cycle testing - When tested in accordance with AAMA 910, there shall be no damage to fasteners, hardware parts, support arms, actuating mechanisms or any other damage causing the window to be inoperable, and air leakage and water resistance tests shall not exceed the Gateway Performance Requirements specified in Table 2.1.

  - Forced entry resistance as per ASTM F 588 = Type A; Grade 10

##### PART 2 – PRODUCTS

2.01 APPROVED MANUFACTURER

Champion 7500 HS-AW-PG65 Horizontal Sliding Window

2.02 MATERIALS

A. 	Aluminum: Aluminum shall be of commercial quality and of proper alloy and temper for window construction, free from defects impairing strength and durability. All extruded sections shall be of 6063-T5 or 6063-T6 and shall have a minimum ultimate tensile strength of 22,000 psi and a yield of 16,000 psi.

B. 	Window Members: Main framing and sash members shall have a nominal wall thickness of not less than 0.080". Main framing and sash members shall have an overall depth of not less than 4.125 inches.  Frame sill shall have a nominal wall thickness of 0.125".

C. 	Fasteners: All screws and other miscellaneous fastening devices incorporated in the product shall be of aluminum, stainless steel or other corrosion-resistant material(s) compatible with aluminum and of sufficient strength to perform the functions for which they are used. Cadmium zinc-plated steel, where used, shall be in accordance with ASTM A 165 or B 633. Nickel or chrome-plated steel, where used, shall be in accordance with ASTM B 456.

D. 	Hardware: Hardware having component parts which are exposed shall be of aluminum, stainless steel or other non corrosive material(s) compatible with aluminum and of sufficient strength to perform the functions for which they are used. Cadmium or zinc-plated steel, where used, shall be in accordance with ASTM A 165 or B 633. Optional: white bronze automatic engagement lock wan keeper for use with the bypass sash option only.

E. 	Weather-stripping: Weather-stripping shall meet the requirements of the specifications as detailed in the appropriate test report. All weather-stripping shall be installed so that there is no metal-to-metal Section 08520 Aluminum Windows - 3 - Series 7500 HS-AW65 HS-HC65 Horizontally Sliding Window contact between the master frame and the sash. All weather-stripping shall be installed in specially extruded ports and secured to prevent movement, shrinkage, or loss when removing sash either for cleaning or repair. Adhered weather-stripping shall not be allowed. Sash of horizontal slider windows shall be weather-stripped using woven pile with mylar center fin, double on sash stiles and single on sash rails.

F. 	Rollers: All rollers shall be tandem type fabricated with a nylon casing and stainless steel rollers. Roller housings shall be cast aluminum.

- Screens: One half screen held in exterior tracks with stainless steel leaf springs; 5/16" x 1 1/2" x .045" extruded tubular aluminum frame; corners mitered, gusset reinforced, and crimped; 18 x 16 dark fiberglass mesh; PVC spline.

2.03 FABRICATION

A. 	Assembly: The windows shall be assembled in a secure and professional manner to perform as herein specified and to assure neat and weather tight construction. All joints of main frames shall be coped and of butt type construction, neatly joined and secured by means of two screws into integral screw ports. All sash corners shall be of coped and butt-type construction, neatly joined and secured by means of two screws anchored into an integral screw port at all horizontal member locations. Meeting stiles of both sash shall mechanically interlock in a closed position. All main framing units shall be sealed with sealants meeting the requirement of AAMA 803 or 809.

B. 	Thermal Barrier: All main frames and ventilators shall be thermally broken by the poured and debridged method. The thermal barrier material shall be of polyurethane fill with ultimate tensile strength to meet or exceed ASTM D 638.

C. 	Glazing: The horizontal sliding aluminum windows shall be glazed with 1" dual sealed, structurally glazed insulated glass.

D. 	Finish: The exposed surfaces of the aluminum members shall be clean and free from serious surface blemishes. The finish is to be electrostatically-applied painted conforming to meet AAMA criteria.

- Frame Options: Extruded Flanges: Head, Sill and Jamb at 2 ½.”

2.04 GLASS AND GLAZING MATERIALS

- Construction: Provide hermetically sealed insulating glass units.  All stainless steel spacers to be continuous with bent corners and containing a dual seal of polyisobutylene and silicone.  Plastic corner keys will not be accepted.

- Exterior glass lite

  - Thickness: 1/4" [Optional: 1/8” or 3/16” may be used, however design and structural performance may vary with thickness].

  - Tint: clear. Optional: (Grey, Bronze, Green)

  - Type: Annealed Optional: (Heat Strengthened, Tempered)

  - Coating: Optional: (Pyrolitic Low-E on #2 surface)

- Interior glass lite

  - Thickness: 1/4" [Optional: 1/8” or 3/16” may be used, however design and structural performance may vary with thickness].

  - Tint: clear. Optional: (Grey, Bronze, Green)

  - Type: Annealed Optional: (Heat Strengthened, Tempered)

  - Coating: Optional:  (Pyrolitic Low-E on #3 surface)

- Seal durability: conformance to ASTM E 774; visible, ALI certification for CBA rating level.

2.05 OTHER GLASS AND GLAZING MATERIALS – ALUMINUM INSULATED PANELS

A.    	1” Insulating Panel with 1/8” hardboard backups and smooth Aluminum Skin on the interior and exterior.  Core to be polystyrene. Options: (Other panel, Spandrel Glass, etc)

2.06 FINISH ON ALUMINUM EXTRUSIONS

- Metal Application: Apply finish on clean extrusions free from serious surface blemishes or scratches.  Finish exposed surfaces visible when the installed product's operating sash is closed.

B.    	Finish to be PPG Duracron, which meets AAMA 2603. [Optional: Acranar 50% Kynar AAMA 2604, Duranar 70% Kynar AAMA 2605, Duranar XL]

C.    	Quality standard: conforming to AAMA 2603. [Optional: 2604, 2605]

D.    	Pretreatment: five-stage; zinc chromate conversion coating.

E.    	Application: an approved applicator using a factory-based electrostatic spray and oven bake system.

F.    	Coating quantity: one color coat.

G.    	Dry film thickness: minimum .7 mils [Optional: 1.4 mil Acranar, 1.2 mils Duranar] on exposed surfaces, except inside corners and channels.

H.    	Color: chosen from manufacturer's standards. [Optional: custom color to be selected.]

Standard Clear Anodized Finish:

- AAM12C22A31 Class II (A3) Clear Anodized finish (.4mils).

Optional Anodized Finish:

- AAM12C22A31 Class I (A4) Clear Anodized Finish (.7 mils).

- Color: (Light Bronze) (Medium Bronze) (Dark Bronze) (Black)

2.07 AIR CONDITIONERS

- Provide A/C sashes and A/C kits to the following windows:

- [Optional:  Provide A/C sleeves by a designated manufacturer.  Include Manufacturer’s name and Model number.]

### PART 3 – EXECUTION

3.01 PROJECT SUBMITTALS

Provide project submittals per the following:

- Product Data: Submit manufacturer's specifications, recommendations and standard details for aluminum window units, including certified test laboratory reports as necessary to show compliance with requirements.

- Shop Drawings:  Submit shop drawings, including location floor plans or exterior wall elevations showing all window openings, typical unit elevations, and full size detail sections of every typical composite member.  Indicate the type of anchors, hardware, operators and other components not included in manufacturer's standard data.  Include glazing details and standards for factory glazed units.

C.   	Samples:   Submit three samples of each required aluminum finish on a metal panel sample or a 6-inch long section of extrusion.

3.02 DELIVERY, STORAGE, AND HANDLING

- Handle all windows and accessories in accordance with AAMA CW-10.

- Protect the windows and accessories from the elements, construction activities, and other hazards until the project is complete.

3.03 PROJECT SITE INSPECTION

- Field verify that the existing window openings are within tolerance, plumb, level, clean, and provide a solid anchoring surface and substrate.  Also confirm that the openings and are in accordance with approved shop drawings.

3.04 INSTALLATION

- Install all of the Aluminum windows and accessories with skilled installers in accordance with all of the manufacturer's recommendations, project documents, and the approved shop drawings.

- All window openings must be covered and secure at the end of each workday.

- Provide the required shims and blocking and fasten the frames to the opening.

- Set each window plumb, level and square, without twisting or bowing the frames.

- Pack fibrous insulation into voids at the window frame perimeter as requested on the architectural drawings.

- Apply sealant at joints and all required metal to metal intersections and the window perimeter as required.  Follow the sealant manufacturer’s recommendations listed in their manufacturer’s data sheets.

3.05 DISPOSAL OF DEBRIS

- Remove all garbage off site and legally dispose of existing windows and debris generated from the installation of the new windows.

3.06 OPTIONAL FIELD TESTING

- Conduct all on-site testing of installed units in conformance with AAMA publication number 502 Voluntary Specification for Field Testing of Windows and Sliding Glass Doors.  Conduct air and water infiltration testing with the window manufacturer, contractor, and owner present.

- An AAMA accredited lab will be hired by the owner to perform the required testing.

3.07 ADJUSTMENT AND CLEAN UP

- Adjust all products, sash, vents, and hardware after installation, as necessary to provide proper operation and a weather tight installation

- Remove any labels and dirt from the window.

# END OF SECTION 085113